ANNEX to WGT/02/07

Proposed amendments in the XML Reference Guide

for the introduction of the new

Request/ Response message

### Get ShipCall notification(s) details

#### Overview

| ##### Introduction | A Member State may ask SafeSeaNet: - To get specific operational information related to the various stages of a ship call for a given vessel. Such information may relate, for instance, to the expected ship call on the basis of the ETA, the most recent departure of the ship or on the basis of ATD, etc.; - To get the whole set of notifications concerning the current voyage of a specific ship (i.e. the pre-arrival, arrival, departure notifications and associated – if made available – Hazmat declarations); - To get information on ship calls at a specific European Port during a specific time frame; - To get confirmation that Hazmat material exist on board a ship during her current voyage; - To get a short summary of Hazmat declaration describing the dangerous good on board during her current voyage; - To get the whole set of information made available by a M.S on dangerous goods carried on board a ship during her current voyage; Such service is implemented by exchanging XML messages between the data requester, the SafeSeaNet system and (in case of Hazmat summary or detailed information) with the data provider The messages are used by the “Information Requests” process agreed for SSN This section describes the different XML messages provided for this transaction Note: The request/ response messages described in this section can be expanded – by adding the relevant elements - at a later stage to include additional blocks for providing to requesters information included in the waste and security notifications concerning the current voyage of specific ship. Further it might be expanded (by adding the relevant elements) to incorporate the request/ response for MRS notification and incident notifications for identified ships |
| --- | --- |

### Get ShipCall notification(s) details

#### Overview

| ##### General flow of the XML messages | The following figure outlines the expected asynchronous flow of XML messages related to this SafeSeaNet XML transaction. You may notice that, as SafeSeaNet has most of ships calls’ related details in its database (stored when e.g. receiving the MS2SSN_PortPlus_Not.xml notification messages from the data provider during the various phases of a ship voyage), there is no need to ask the data provider for sending details, except in the case of the request of the Hazmat details |
| --- | --- |

| NCA_A (Data Requester) |  | SafeSeaNet |  |  |  | NCA_B (Data Provider) |
| --- | --- | --- | --- | --- | --- | --- |
| MS2SSN_ShipCall_Req ACK(HTTP 202 return code) |  |  |  |  |  |  |
| SSN_Receipt |  |  |  | SSN2MS_ShipCall _Req ACK (HTTP 202 return code) |  |  |
|  |  |  |  | MS2SSN_ ShipCall_Res ACK (HTTP 202 return code) SSN_Receipt |  |  |
| SSN2MS_ShipCall_Res ACK (HTTP 202 return code) |  |  |  |  |  |  |

| ##### Contents | This section contains the following topics: |
| --- | --- |

Topic

MS2SSN_ShipCall_Req.xml message structure MS2SSN_ShipCall_Req.xml message applicable business rules Examples

SSN2MS_ShipCall_Req.xml message structure SSN2MS_ShipCall_Req.xml message applicable business rules Examples

MS2SSN_ShipCall_Res.xml message structure MS2SSN_ShipCall_Res.xml message applicable business rules Examples

SSN2MS_ShipCall_Res.xml message structure SSN2MS_ShipCall_Res.xml message applicable business rules Examples

Detailed description of the attributes included in the messages (type/ length/ definition/ rules applicable to type or length) are provided in the ANNEX B of the XML RG

#### MS2SSN_ShipCall_Req.xml message

| ##### Introduction | The MS2SSN_ShipCall_Req.xml message is sent by a Member State (data requester) to SafeSeaNet in order to request the operational information (ship specific/ ship call specific/ port of call specific). Please note that such kind of XML request (MS2SSN_<SSN_Tx_Type>_Req.xml) and its corresponding XML response (SSN2MS_<SSN_Tx_Type>_Res.xml) should only be implemented by a Member State if it wants to develop its own data requester interface instead of using the default browser-based web interface supplied by SSN. |
| --- | --- |

| ##### Message description | The following table gives an overview of the XML message used for the transaction and the applicable business rules. The detailed definition of the attributes is included in the ANNEX B of the XML RG |
| --- | --- |

| PortPlus Message - MS2SSN_ShipCall_Req.xml message |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Header (1) |  |  |  | Version TestId MSRefId SentAt TimeoutValue From To | 1 0-1 1 1 1 1 1 |
| Body (1) |  |  |  |  |  |
|  | RequiredResponseCriteria (1) | ShipCallResp (1) |  | GetDetails GetHazMat <TBD1> <TBD2>1 <TBD3>1 | 1 0-1 |
|  |  | <TBD> (0-1) |  | <TBD1> <TBD2> <……> <TBDx> |  |
|  |  | SearchCriteria (1) | TimePeriodCriteria (1) | StartDateTime EndDateTime | 1 1 |
|  |  |  | ShipIdentificationCriteria (0-1) | IMONumber MMSINumber CallSign ShipName | 0-1 0-1 0-1 0-1 |
|  |  |  | PortOfCallIdentificationCriteria (0-1) | PortOfCall <TBD> | 1 |
|  |  |  | AdditionalSearchCriteria (0-1) | Flag <TBD> | 0-99 |
|  |  |  | <TBD> |  |  |

#### MS2SSN_ShipCall_Req.xml message

| ##### Business rules | The following table describes the XML message used for the transaction and the applicable business rules. The detailed definition of the attributes is included in the ANNEX B of the XML RG |
| --- | --- |

| Item | Occ | Business rule |
| --- | --- | --- |
| Header | 1 | Header Node |
| Version | 1 | None |
| TestId | 0-1 | None |
| MSRefId | 1 | The MSRefId must be unique |
| SentAt | 1 | None |
| TimeoutValue | 1 | None |
| From | 1 | Standard practice for the field is to include the reference identification of the originator of the data included in the message |
| To | 1 | None |
| Body | 1 | Body Node |
| RequiredResponseCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given |
| ShipCallResp | 0-1 | ElementNode (child to RequiredResponse Criteria) defining the business rules related to the anticipated response – Child to RequiredResponseCriteria |
| GetDetails | 1 | Refer to the table “xx1” where the applicable business logic for the query related to each value is described |
| GetHazMat | 0-1 | If the GetDetails attribute above has one of the following values: - ExpectedCallOfSelectedShip - MostRecentArrivalOfSelectedShip - MostRecentDeparture OfSelectedShip - LatestregisteredShipCallDataOfSpecificShip then the OCC of this attribute can only be set as OCC=1. Should “HazMatDetails” is specified as the value of this attribute the details provided in the response might differ depending on the capability of the data provider to communicate details via XML or not (refer to the business rules applicable to the response message): |
| <TBD1>1 |  | TBD rules |
| <TBD2> |  | TBD rules |
| <TBD3> |  | TBD rules |
| <TBD>2 | 0-1 | ElementNode (child to RequiredResponseCriteria) defining the business rules related to the anticipated response – Child to RequiredResponseCriteria |
| <TBD1> |  | TBD rules |
| <TBD2> |  | TBD rules |
| …… |  | TBD rules |
| …… |  | TBD rules |
| <TBDx> |  | TBD rules |
| SearchCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given – Child to RequiredResponseCriteria |
| ShipIdentificationCriteria | 0-1 | ShipIdentification-based search criteria element node – Child to Search Criteria |
| IMONumber | 0-1 | Refer to business rules table “xx1” identifying when this attribute has to be provided by the data requester. |
| MMSINumber | 0-1 | None |
| CallSign | 0-1 | None |
| ShipName | 0-1 | None |
| PortOfCallIdentificationCriteria | 0-1 | VoyageIdentification-based search criteria element node - – Child to search criteria |
| PortOfCall | 1 | Refer to business rules table “xx1” identifying when this attribute has to be provided by the data requester |
| TimePeriodCriteria | 1 | TimePeriod-based search criteria element node - – Child to search criteria element |
| StartDateTime | 1 | Refer to business rules table “xx1” identifying when this attribute has to be provided by the data requester and the maximum length of period |
| EndDateTime | 1 | None |
| AdditionalSearchCriteria | 0-1 | AdditionalSearchCriteria - Child to search criteria element |
| Flag | 0-99 | Should this attribute is provided the results of the query will be limited to ships |
| <tbd1> |  | Tbd rules |
| … |  | Tbd rules |
| … |  | Tbd rules |
| <tbd..xx> |  | Tbd rules |

Table xx1 – Business rules for the queries identified by the GetDetails attribute

| Nr. | Value of the GetDetails attribute in the MS2SSN_PortPlusReq_msg | Information expected to be included in the response message | Attributes than need to be set, at minimum, for accepting the request message as valid | Query will return results related to these rules |
| --- | --- | --- | --- | --- |
| 1 | ExpectedCallOfSelectedShip | Data relevant to expected calls of a ship (on the basis of ETA) The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification listed by the query: - VesselIdentification - VoyageInformation - 3DaysPreArrivalNotification Details - 24hoursPreArrivalNotification Details | (IMONumber or MMSINumber) StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester” EndDateTime = “StartDateTime” Rules The SSN EIS shall first identify all the Ship Calls for the specified vessel with ETAtoPortOfCall “in the future” with respect to the StartDateTime set by the data requester. Then, the query shall list the ShipCall with ETAtoPortOfCall closer to the StartDateTime The applicable rules are: [DateTime=”SentAt” of the request] <=StartDateTime<=[DateTime=”SentAt” of the request” plus 30 Days] Last update of MIN (ETAtoPortOfCall) from pre-arrival notifications to be listed >= [Datetime=” StartDateTime”] |
| 2 | MostRecentArrivalOfSelectedShip | Search for the most recent arrival of a ship (on the basis of ATA) The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification listed by the query: - VesselIdentification - VoyageInformation - ArrivalNotification Details | (IMONumber or MMSINumber) StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester” EndDateTime = “StartDateTime” Rules The SSN EIS shall first identify all the Ship Calls for the specified vessel with ATA “in the past” with respect to the StartDateTime set by the data requester. Then, the query shall list the ShipCall with ATA closer to the StartDateTime The applicable rules are: [DateTime=”SentAt” of the request] minus 30 Days <=StartDateTime<=[DateTime=”SentAt] MAX(ATA)<= “StartDateTime”, AND (if available) the [ETDfromPortOfCall or ATD(specified in the PortPlus notification that gives the Max (ATA))>= StartDateTime] |
| 3 | MostRecentDepartureOfSelectedShip | Search for the most recent departure notification of a vessel (on the basis of ATD) The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification listed by the query: - VesselIdentification - VoyageInformation - DepartureNotification Details | (IMONumber or MMSINumber) StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester” EndDateTime = “StartDateTime” Rules The SSN EIS shall first identify all the Ship Calls for the specified vessel with ATD “in the past” with respect to the StartDateTime set by the data requester. Then, the query shall list the ShipCall with ATD closer to the StartDateTime The applicable rules are: [DateTime=”SentAt” of the request] minus 30 Days <=StartDateTime<=[DateTime=”SentAt] [MAX (ATD) from PortPlus notification <= StartDateTime] |
| 4 | RecentAndCurrentShipCallsOfSelectedShip | Search for ship calls of a given vessel at a specific (selectable) timeframe within the last 30 days on the basis of ATA The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification (s) listed by the query: - VesselIdentification - VoyageInformation - 3DaysPreArrivalNotification Details - 24hoursPreArrivalNotification Details - ArrivalNotification Details (if available) | (IMONumber or MMSINumber) StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester” EndDateTime = “A DateTime set by the requester” Rules The SSN EIS shall first identify all the Ship Calls for the specified vessel with ATA “in the past” with respect to the StartDateTime set by the data requester and within the period specified by the StartDateTime and EndDateTime.. The query shall return the data of these Ship Calls The calendar period between StartDateTime and EndDateTime cannot be more than 30 days [DateTime=”SentAt” of the request] minus 30 Days <=StartDateTime<=[DateTime=”SentAt] [ATA from PortPlus notification <= StartDateTime] |
| 5 | ExpectedShipCallsAtEUport | Search for expected Ship Calls at a given (to be specified) port for a specific (selectable) timeframe within the next 30 days (on the basis of ETA) The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification (s) listed by the query: - VesselIdentification - VoyageInformation - 3DaysPreArrivalNotification Details - 24hoursPreArrivalNotification Details | PortOfCall StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester” EndDateTime = “A DateTime set by the requester” within the next 30 calendar days from the StartDateTime Rules The SSN EIS shall first identify all the Ship Calls for the specified vessel with ETAtoPortOfCall “in the future” with respect to the StartDateTime set by the data requester and within the period specified by the StartDateTime and EndDateTime.. Query shall list only those PortPlus notifications related to ship calls where the ATA value is still not specified by the data providers Value ZZUKN for PortOfCall is not allowed [DateTime=”SentAt” of the request] plus 30 Days .>=StartDateTime.>=[DateTime=”SentAt] [ETAtoPortOfCall from PortPlus notification >= StartDateTime] |
| 6 | CurrentShipCallsAtEUport | Search for current Ship Calls at a given port (Calls with ATA within the last 24 hours or less) within a timeframe in the last 24 hours The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification (s) listed by the query: - VesselIdentification - VoyageInformation - Details - ArrivalNotification Details | PortOfCall StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester within the last 24 hours” EndDateTime = “A DateTime set by the requester within the last 24 hours” Rules The SSN EIS shall first identify all the Ship Calls for the specified vessel with ATA “in the past” with respect to the StartDateTime set by the data requester and within the period specified by the StartDateTime and EndDateTime.. The Ship Calls data will be listed by the query. Value ZZUKN for PortOfCall is not allowed Query shall list only those PortPlus notifications related to ship calls where the ATA value is known. [DateTime=”SentAt” of the request] minus 24 hours <=StartDateTime<=DateTime=”SentAt] [DateTime=”SentAt” of the request] minus 24 hours <=EndDateTime<=DateTime=”SentAt] [StartDateTime>=ATA from PortPlus notification >= EndDateTime] AND [EDfromPortOfCall (if available) or ATD >=EndTimeDate] |
| 7 | CompletedShipCallsAtEUport | Search for completed Ship Calls at a given (to be specified) port for a specific (selectable) timeframe within the last 30 days on the basis of ATD The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification (s) listed by the query: - VesselIdentification - VoyageInformation - DepartureNotification Details | PortOfCall StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester within the last 30 days” EndDateTime = “A DateTime set by the requester within the last 30 days” Rules The SSN EIS shall first identify all the Ship Calls for the specified vessel with ATD “in the past” with respect to the StartDateTime set by the data requester and within the period specified by the StartDateTime and EndDateTime.. The Ship Calls data will be listed by the query. Value ZZUKN for PortOfCall is not allowed Query shall list only those PortPlus notifications related to ship calls where the ATD value is known. [DateTime=”SentAt” of the request] minus 30 days <=StartDateTime<=DateTime=”SentAt] [DateTime=”SentAt” of the request] minus 30 days <=EndDateTime<=DateTime=”SentAt] [StartDateTime>=ATD from PortPlus notification >= EndDateTime] |
| 8 | LatestRegisteredShipCallDataOfSpecificShip | Provision of the ship call information for the Ship Call last registered to SSN EIS (considering the “SentAt”of the PortPlus notification with respect to the “SentAt” datetime of the request message The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification (s) listed by the query: - VesselIdentification - VoyageInformation - 3DaysPreArrivalNotification Details - 24hoursPreArrivalNotification Details - ArrivalNotification Details - DepartureNotification Details | (IMONumber or MMSINumber) StartDateTime EndDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “SentAt of the request” EndDateTime = “SentAt of the request” Rules [Max (SentAt) of the listed PortPlus notification<= StartDateTime] |
| 9 | PortPlusNotificationsReceivedbySSNyesterday | Provision of a list of data related to all the ship calls registered in SSN within the previous calendar date from the date specified in the StartDateTime attribute. The provision of data will be based on a pre-calculated query The query shall return a confirmation on dangerous goods availability on board as well as the data included in the following specific elements of the PortPlus notification (s) listed by the query: - VesselIdentification - VoyageInformation - 3DaysPreArrivalNotification Details - 24hoursPreArrivalNotification Details - ArrivalNotification Details - DepartureNotification Details | StartDateTime | Values set for Start DateTime - EndDateTime StartDateTime = “A DateTime set by the requester within the last 30 days” |

| ##### Examples of request messages | The following examples illustrates the details of request messages of the MS2SSN_ShipCall_Req.xml type: Example in XML format to be included |
| --- | --- |

#### SSN2MS_ShipCall_Req.xml message

| ##### Introduction | The SSN2MS_ShipCall_Req.xml message is sent by SafeSeaNet to the Member State owning the details requested (data provider) in order to request the latest relevant notification details (e.g. HazMat) about a given vessel. This message is used by SafeSeaNet when receiving a MS2SSN_ShipCall_Req.xml message coming from a data requester and when SafeSeaNet has identified that the data provider (i.e. the owner of the notification details) is able to talk XML with SafeSeaNet (please refer to “xxxx” at page yyy for more details). The data provider must have implemented this XML message and its XML response accordingly. Please note that such kind of XML request (SSN2MS_<SSN_Tx_Type>_Req.xml) and its corresponding XML response (MS2SSN_<SSN_Tx_Type>_Res.xml) must be implemented by a Member State (data provider) in order to supply the notification details in XML format. |
| --- | --- |

| ##### Message description | The following table describes the XML message used for the transaction. |
| --- | --- |

| PortPlus Message - SSN2MS_ShipCall_Req.xml message |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Header (1) |  |  |  | Version TestId SSNRefld SentAt TimeoutValue From To | 1 0-1 1 1 1 1 1 |
| Body (1) |  |  |  |  |  |
|  | RequiredResponseCriteria (1) | ShipCallResp (0-1) |  | GetHazMat <TBD1> <TBD2> <TBD3> | 1 |
|  |  | <TBD> (0-1) |  | <TBD1> <TBD2> <……> <TBDx> |  |
|  |  |  | ShipIdentificationCriteria (0-1) | IMONumber MMSINumber CallSign ShipName | 0-1 0-1 0-1 0-1 |
|  |  |  | AdditionalSearchCriteria (0-1) | ShipCallID <TBD> | 0-1 |
|  |  |  | <TBD> |  |  |

#### SSN2MS_ShipCall_Req.xml message

| ##### Business rules | The following rules apply. |
| --- | --- |

| Item | Occ | Business rule |
| --- | --- | --- |
| Header | 1 | Header Node |
| Version | 1 | None |
| TestId | 0-1 | None |
| SSNRefId | 1 | The SSNRefId must be unique |
| SentAt | 1 | None |
| TimeoutValue | 1 | None |
| From | 1 | None |
| To | 1 | None |
| Body | 1 | Body Node |
| RequiredResponseCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given |
| ShipCallResp | 0-1 | ElementNode (child to RequiredResponseCriteria) defining the business rules related to the anticipated response – Child to RequiredResponseCriteria |
| GetHazMat | 0-1 | The response message should include the hazmat information of the specific ShipCall identified by the value of the attribute under AdditionalShipCriteria below. |
| <TBD1> |  | TBD rules |
| <TBD2>1 |  | TBD rules |
| <TBD3>1 |  | TBD rules |
| <TBD> | 0-1 | ElementNode (child to RequiredResponseCriteria) defining the business rules related to the anticipated response – Child to RequiredResponseCriteria |
| <TBD1> |  | TBD rules |
| <TBD2>2 |  | TBD rules |
| … |  | TBD rules |
| ….. |  | TBD rules |
| <TBDx>2 |  | TBD rules |
| SearchCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given - Child to RequiredResponseCriteria |
| ShipIdentificationCriteria | 0-1 | ShipIdentification-based search criteria element node – Child to search criteria |
| IMONumber | 0-1 | Refer to business rules table “xx1” dentifying when this attribute has to be provided by the data requester |
| MMSINumber | 0-1 |  |
| CallSign | 0-1 |  |
| ShipName | 0-1 |  |
| AdditionalSearchCriteria | 0-1 | AdditionalSearchCriteria - Child to Search Criteria element |
| ShipCallID | 0-1 | The reference identifier of the ship call for which details are requested Mandatory in case the request message relates with provision of Hazmat details |
| <tbd1> |  | Tbd rules |
| …. |  | Tbd rules |
| ……. |  | Tbd rules |
| <tbd..xx> |  | Tbd rules |

| ##### Examples of request messages | The following examples illustrates the details of request messages of the SSN2MS_ShipCall_Req.xml type: Example in XML format to be included |
| --- | --- |

#### MS2SSN_ShipCall_Res.xml message

| ##### Introduction | The MS2SSN_ShipCall_Res.xml message is sent by the Member State owning the notifications details (data provider) to SafeSeaNet in answer to its request for getting the relevant, to the request made, notification details (e.g. HazMat) about a given vessel. Please note that such kind of XML response (MS2SSN_<SSN_Tx_Type>_Res.xml) and its corresponding XML request (SSN2MS_<SSN_Tx_Type>_Req.xml) must be implemented by a Member State (data provider) in order to supply the notification details in XML format |
| --- | --- |

| ##### Structure of the response message | The following table describes the XML message used for the transaction. Note that the location of the cargo manifest has been given at notification time (see xxxx at p. yyyy): |
| --- | --- |

| PortPlus Message –MS2SSN_ShipCall_res.xml |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header (1) |  |  |  |  |  |  |  | Version TestId MSRefId SSNRefld SentAt TimeoutValue From To StatusCode StatusMessage | 1 0-1 1 1 1 1 1 1 1 0-1 |
| Body (1) |  |  |  |  |  |  |  |  |  |
|  | ProvidedResponseCriteria (1) | ShipCallResp (0-1) |  |  |  |  |  | GetHazMat <TBD1> <TBD2> <TBD3> | 1 |
|  |  | <TBD> (0-1) |  |  |  |  |  | <TBD1> <TBD2> <……> <TBDx> |  |
|  |  |  | ShipIdentificationCriteria (0-1) |  |  |  |  | IMONumber MMSINumber CallSign ShipName | 0-1 0-1 0-1 0-1 |
|  |  |  | AdditionalSearchCriteria (0-1) |  |  |  |  | ShipCallID <TBD> | 1 |
|  | QueryResults (0-1) | VesselIdentification (1) |  |  |  |  |  | IMONumber MMSINumber CallSign ShipName Flag | 0-1 0-1 0-1 0-1 0-1 |
|  |  | VoyageInformation (1) |  |  |  |  |  | ShipCallID LastPort PortOfCall PositionInPortOfCall ETDfromLastPort ETAtoPortOfCall ETDfromPortOfCall NextPort ETANextPort PoBasinHazmatNotification | 1 0-1 0-1 0-1 0-1 0-1 0-1 0-1 0-1 0-1 |
|  |  | HazmatInformation (0-1) | HazmatSummary (0-1) |  |  |  |  | DGclassification INFShipClass PoBasinHazmatNotification < TBD1> | 0-∞ 0-1 1 |
|  |  |  | HazmatNotificationDetailsViaXML(0-1) | CargoInformation (1) | PlacementOfGoods (0-99) |  |  | LocationOnBoardGoods | 1 |
|  |  |  |  |  |  | DPG (1-9999) |  | TechnicalName IMOHazardClass UNNumber | 1 1 1 |
|  |  |  |  |  |  |  | WeightGross (0-1) | UnitOfMeasurementGross GrossQuantity | 1 1 |
|  |  |  |  |  |  |  | WeightNet (0-1) | UnitOfMeasurementNet NetQuantity | 1 1 |
|  |  |  |  |  | PlacementOfGoodsInContainer (0-99) |  |  | TransUnitId LocationOnBoardContainer | 1 1 |
|  |  |  |  |  |  | DPG (1-9999) |  | TechnicalName UNNumber IMOHazardClass | 1 1 1 |
|  |  |  |  |  |  |  | WeightGross (0-1) | UnitOfMeasurementGross GrossQuantity | 1 1 |
|  |  |  |  |  |  |  | WeightNet (0-1) | UnitOfMeasurementNet NetQuantity | 1 1 |
|  |  | TBD |  |  |  |  |  | < TBD1> |  |

MS2SSN_ShipCall_Res.xml message

| ##### Business rules | The following business rules apply: |
| --- | --- |

| Item | Occ | Description |
| --- | --- | --- |
| Header | 1 | Header Node |
| Version | 1 | None |
| TestId | 0-1 | None |
| MSRefId | 1 | The MSRefId must be unique |
| SSNRefld | 1 | None |
| SentAt | 1 | None |
| TimeoutValue | 1 | None |
| From | 1 | None |
| To | 1 | None |
| StatusCode | 1 | None |
| StatusMessage | 1 | None |
| Body | 0-1 | Body Node (only optional when StatusCode=”InvalidFormat”) |
| ProvidedResponseCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given |
| ShipCallResp | 0-1 | ElementNode (child to RequiredResponseCriteria) defining the business rules related to the anticipated response – Child to ProvidedResponseCriteria |
| GetHazMat | 0-1 | From original MS2SSN_ShipCall_Req.xml request |
| <TBD1> |  |  |
| <TBD2>1 |  |  |
| <TBD3>1 |  |  |
| <TBD> | 0-1 | Element Node (child to ProvidedResponseCriteria) defining the business rules related to the anticipated response – Child to ProvidedResponseCriteria |
| <TBD1> |  | From original MS2SSN_ShipCall_Req.xml request |
| <TBD2>2 |  |  |
| … |  |  |
| … |  |  |
| <TBDx>2 |  |  |
| SearchCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given - Child to ProvidedResponseCriteria |
| ShipIdentificationCriteria | 0-1 | ShipIdentification-based search criteria element node – Child to search criteria |
| IMONumber | 0-1 | From original MS2SSN_ShipCall_Req.xml request |
| MMSINumber | 0-1 |  |
| CallSign | 0-1 |  |
| ShipName | 0-1 |  |
| AdditionalSearchCriteria | 0-1 | Additional Search criteria element node – Child to search criteria |
| ShipCallID | 0-1 | From original MS2SSN_ShipCall_Req.xml request |
| <tbd1> |  |  |
| QueryResults | 0-1 | Element node, child to Body, including the results of the query to be executed according to the rules and criteria in the ShipCallResp element of the MS2SSN_ShipCall_Req.xml message. The QueryResults element is not to be included in the response only in case the query does not have any results to return |
| VesselIdentification | 1 | VesselIdentification element node (“child” to the QueryResults element) |
| IMONumber | 0-1 | latest update of the attribute registered in NCA database for the PortPlus notification identified by the ShipCallID |
| MMSINumber | 0-1 |  |
| CallSign | 0-1 |  |
| ShipName | 0-1 |  |
| Flag | 0-1 |  |
| VoyageInformation | 1 | VoyageInformation element (“child” to the QueryResults element |
| ShipCallID | 1 | Reference identifier of the record included in the list returned by the query. |
| LastPort | 0-1 | latest update of the attribute registered in NCA database for the PortPlus notification identified by the ShipCallID |
| PortOfCall | 0-1 |  |
| PositionInPortOfCall | 0-1 |  |
| ETDfromLastPort | 0-1 |  |
| ETAtoPortOfCall | 0-1 |  |
| ETDfromPortOfCall | 0-1 |  |
| NextPort | 0-1 |  |
| ETANextPort | 0-1 |  |
| PoBasinHazmatNotification | 0-1 |  |
| HazmatInformation | 0-1 | HazmatInformation element node – child to QueryResults element |
| HazmatSummary | 0-1 | HazmatCargoInformation second level-element node (“child” to (“child” to the HazmatInformation element) |
| DGclassification | 0-∞ | latest update of the attribute registered in NCA database for the voyage reported using the above mentioned ShipCallID |
| INFShipClass | 0-1 |  |
| < TBD1 |  |  |
| < TBDx> |  |  |
| HazmatNotificationDetailsViaXML | 0-1 | HazmatNotificationDetailsViaXML element node(“child” to the HazmatInformation element) Mandatory if HazmatNotificationDetailsViaPhoneFaxOrURL not provided |
| CargoInformation | 1 | CargoInformation element node – Child to HazmatNotificationDetailsViaXML |
| PlacementOfGoods | 0-99 | PlacementOfGoods element describing the location of goods which are not in containers – child of CargoInformation |
| LocationOnBoardGoods | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| DPG | 1-9999 | DPG element node describing each type of dangerous goods on board - child of CargoInformation |
| TechnicalName | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| UNNumber | 1 |  |
| IMOHazardClass | 1 |  |
| WeightGross | 0-1 | WeightGross element specifying the gross weight of the dangerous good. Mandatory if WeightNet not present. – Child of DPG |
| UnitOfMeasurementGross | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| GrossQuantity | 1 |  |
| WeightNet | 0-1 | WeightNet element specifying the net weight of the dangerous good. Mandatory if WeightGross not present. - – Child of DPG |
| UnitOfMeasurementNet | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| NetQuantity | 1 |  |
| PlacementOfGoodsInContainer | 0-99 | PlacementOfGoodsInContainer element specifying the placement of the container which contains the dangerous good – child of CargoInformation |
| TransUnitId | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| LocationOnBoardContainer | 1 |  |
| DPG | 1-9999 | DPG element node describing each type of dangerous goods on board. – Child of PlacementOfGoodsInContainer |
| TechnicalName | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| UNNumber | 1 |  |
| IMOHazardClass | 1 |  |
| WeightGross | 0-1 | WeightGross element specifying the gross weight of the dangerous good. Mandatory if WeightNet not present. – Child of DPG |
| UnitOfMeasurementGross | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| GrossQuantity | 1 |  |
| WeightNet | 0-1 | WeightNet element specifying the net weight of the dangerous good. Mandatory if WeightGross not present. – Child of DPG |
| UnitOfMeasurementNet | 1 | latest update of the attribute registered in NCA database for the Hazmat declaration notified to SSN EIS using the above mentioned ShipCallID |
| NetQuantity | 1 |  |

| ##### Examples of response messages | The following examples illustrates the details of request messages of the MS2SSN_ShipCall_Res.xml type: |
| --- | --- |

Example in XML format to be included

#### SSN2MS_ShipCall_Res.xml message

| ##### Introduction | The SSN2MS_ShipCall_Res.xml message is the final response sent by SafeSeaNet to a Member State requesting the operational information stored in SSN (at EIS or national level;) about a given vessel, a given port, a specified time period and/or a combination of the three (data requester). Please note that such kind of XML response (SSN2MS_<SSN_Tx_Type>_Res.xml) and its corresponding XML request (MS2SSN_<SSN_Tx_Type>_Req.xml) should only be implemented by a Member State if it wants to develop its own data requester interface instead of using the default browser-based web interface supplied by SSN. |
| --- | --- |

| ##### Structure of the response message | The format of the response message is described below: |
| --- | --- |

| PortPlus Message – SSN2MS_ShipCall_res.xml |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header (1) |  |  |  |  |  |  |  | Version TestId MSRefId SSNRefld SentAt TimeoutValue From To StatusCode StatusMessage | 1 0-1 1 1 1 1 1 1 1 0-1 |
| Body (1) |  |  |  |  |  |  |  |  |  |
|  | ProvidedResponseCriteria (1) | ShipCallResp (0-1) |  |  |  |  |  | GetHazMat <TBD1> <TBD2> <TBD3> | 1 |
|  |  | <TBD> (0-1) |  |  |  |  |  | <TBD1> <TBD2> <……> <TBDx> |  |
|  |  | SearchCriteria (1) | TimePeriodCriteria (1) |  |  |  |  | StartDateTime EndDateTime | 1 1 |
|  |  |  | ShipIdentificationCriteria (0-1) |  |  |  |  | IMONumber MMSINumber CallSign ShipName | 0-1 0-1 0-1 0-1 |
|  |  |  | PortOfCallIdentificationCriteria (0-1) |  |  |  |  | PortOfCall <TBD> | 1 |
|  |  |  | AdditionalSearchCriteria (0-1) |  |  |  |  | Flag <TBD> | 0-1 0-1 |
|  | QueryResults (0-1) | VesselIdentification (1) |  |  |  |  |  | IMONumber MMSINumber CallSign ShipName Flag | 0-1 0-1 0-1 0-1 0-1 |
|  |  | VoyageInformation (1) |  |  |  |  |  | ShipCallID LastPort PortOfCall PositionInPortOfCall ETDfromLastPort ETAtoPortOfCall ETDfromPortOfCall NextPort ETANextPort PoBasinHazmatNotification | 1 0-1 0-1 0-1 0-1 0-1 0-1 0-1 0-1 0-1 |
|  |  | HazmatInformation (0-1) | HazmatSummary (0-1) |  |  |  |  | DGclassification INFShipClass PoBasinHazmatNotification < TBD1> | 0-∞ 0-1 1 |
|  |  |  | HazmatNotificationDetailsViaXML(0-1) | CargoInformation (1) | PlacementOfGoods (0-99) |  |  | LocationOnBoardGoods | 1 |
|  |  |  |  |  |  | DPG (1-9999) |  | TechnicalName IMOHazardClass UNNumber | 1 1 1 |
|  |  |  |  |  |  |  | WeightGross (0-1) | UnitOfMeasurementGross GrossQuantity | 1 1 |
|  |  |  |  |  |  |  | WeightNet (0-1) | UnitOfMeasurementNet NetQuantity | 1 1 |
|  |  |  |  |  | PlacementOfGoodsInContainer (0-99) |  |  | TransUnitId LocationOnBoardContainer | 1 1 |
|  |  |  |  |  |  | DPG (1-9999) |  | TechnicalName UNNumber IMOHazardClass | 1 1 1 |
|  |  |  |  |  |  |  | WeightGross (0-1) | UnitOfMeasurementGross GrossQuantity | 1 1 |
|  |  |  |  |  |  |  | WeightNet (0-1) | UnitOfMeasurementNet NetQuantity | 1 1 |
|  |  | TBD |  |  |  |  |  | < TBD1> |  |

#### SSN2MS_ShipCall_Res.xml message

| ##### Business rules | The business rules in this section apply: Should the data requester requested the provision of Hazmat declaration details, depending on the data provider capabilities (see p. yyyy, the response message shall return: either the details in XML or a URL for the relevant declaration or contact (phone/ fax) details, as indicated in the table xx2 below |
| --- | --- |

| If the data provider… | Then the XML message contains the following node… |
| --- | --- |
| is able to talk XML with SafeSeaNet | … <HazmatInformation….> < HazmatNotificationDetailsViaXML…> </ HazmatInformation …> … |
| can only provide notification details as downloadable files | … <HazmatInformation….> < HazmatNotificationDetailsViaPhoneFaxOrURL …> <UrlDetails…/> </ HazmatInformation …> … |
| is only accessible via phone/fax/email | … <HazmatInformation….> < HazmatNotificationDetailsViaPhoneFaxOrURL …> < ContactDetails …/> </ HazmatInformation …> … |

| Item | Occ | Description |
| --- | --- | --- |
| Header | 1 | Header Node |
| Version | 1 |  |
| TestId | 0-1 |  |
| MSRefId | 1 |  |
| SSNRefld | 1 |  |
| SentAt | 1 |  |
| From | 1 |  |
| To | 1 |  |
| StatusCode | 1 |  |
| StatusMessage | 0-1 |  |
| Body | 0-1 | Body Node (only optional when StatusCode=”InvalidFormat”) |
| ProvidedResponseCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given |
| ShipCallResp | 0-1 | ElementNode (child to RequiredResponseCriteria) defining the business rules related to the anticipated response – Child to ProvidedResponseCriteria |
| GetDetails | 1 | From original MS2SSN_ShipCall_Req.xml request |
| GetHazMat | 0-1 |  |
| <TBD1> |  |  |
| <TBD2>1 |  |  |
| <TBD3>1 |  |  |
| <TBD> | 0-1 | Element Node (child to ProvidedResponseCriteria) defining the business rules related to the anticipated response – Child to ProvidedResponseCriteria |
| <TBD1> |  | From original MS2SSN_ShipCall_Req.xml request |
| <TBD2>2 |  |  |
| … |  |  |
| ….. |  |  |
| <TBDx>2 |  |  |
| SearchCriteria | 1 | SearchCriteria element node(s). Only 1 element node might be given - Child to ProvidedResponseCriteria |
| ShipIdentificationCriteria | 0-1 | ShipIdentification-based search criteria element node – Child to search criteria |
| IMONumber | 0-1 | From original MS2SSN_ShipCall_Req.xml request |
| MMSINumber | 0-1 |  |
| CallSign | 0-1 |  |
| ShipName | 0-1 |  |
| PortOfCallIdentificationCriteria | 0-1 | VoyageIdentification-based search criteria element node - – Child to search criteria |
| PortOfCall | 1 | From original MS2SSN_ShipCall_Req.xml request |
| <TBD> | 1 |  |
| TimePeriodCriteria | 1 | TimePeriod-based search criteria element node - – Child to search criteria e |
| StartDateTime | 1 | From original MS2SSN_ShipCall_Req.xml request |
| EndDateTime | 1 |  |
| AdditionalSearchCriteria | 0-1 |  |
| Flag |  | From original MS2SSN_ShipCall_Req.xml request |
| <tbd1> |  |  |
| QueryResults | 0-1 | Element node, child to Body, including the results of the query to be executed according to the rules and criteria in the ShipCallResp element of the MS2SSN_ShipCall_Req.xml message. The QueryResults element is not to be included in the response only in case the query does not have any results to return |
| PortPlusNotificationDetailsExludingHazMatInfo_LIST | 0-∞ | Element node, child to QueryResults, returning 0-∞ times the details of PortPlus notifications included in a list created on the basis of the value of the GetDetails attribute included in the relevant MS2SSN_ShipCall_Req.xml message. Each record included in the list will provide the latest update of attributes values included in a set of PortPlus notifications stored in SSN EIS corresponding to the unique ShipCallID |
| VesselIdentification | 1 | VesselIdentification element node (“child” to the PortPlusNotificationDetailsExluding HazMatInfo_LIST element) |
| IMONumber | 0-1 | latest update of the attribute registered in SSN EIS notification database for the PortPlus notification identified by the ShipCallID |
| MMSINumber | 0-1 |  |
| CallSign | 0-1 |  |
| ShipName | 0-1 |  |
| Flag | 0-1 |  |
| VoyageInformation | 1 | VoyageInformation element node (“child” to the PortPlusNotificationDetailsExluding HazMatInfo_LIST element) |
| ShipCallID | 1 | Reference identifier of the record included in the list returned by the query. Copies o the unique ShipCallID identifier originally communicated to SSN EIS by the data provider |
| LastPort | 0-1 | latest update of the attribute registered in SSN EIS notification database for the PortPlus notification identified by the ShipCallID |
| PortOfCall | 1 |  |
| PositionInPortOfCall | 0-1 |  |
| ETDfromLastPort | 0-1 |  |
| ETAtoPortOfCall | 0-1 |  |
| ETDfromPortOfCall | 0-1 |  |
| NextPort | 0-1 |  |
| ETANextPort | 0-1 |  |
| 3DaysPreArrivalNotification Details | 0-1 | 3DaysPre-arrivalNotification Details element node. (“child” to the PortPlusNotificationDetailsExludingHazMatInfo_LIST element) |
| PossibleAnchorage | 0-1 | latest update of the attribute registered in SSN EIS notification database for the PortPlus notification identified by the ShipCallID |
| PlannedOperations | 0-1 |  |
| PlannedWorks | 0-1 |  |
| ShipConfiguration | 0-1 |  |
| CargoVolumeNature | 0-1 |  |
| ConditionCargoBallastTanks | 0-1 |  |
| 24hoursPreArrivalNotification Details | 0-1 | 24hours Pre-arrivalNotification Details element node. (“child” to the PortPlusNotificationDetailsExludingHazMatInfo_LIST element) |
| POBvoyagetowardsPortOfCall | 1 | latest update of the attribute registered in SSN EIS notification database for the PortPlus notification identified by the ShipCallID |
| ArrivalNotification Details | 0-1 | ArrivalNotification Details element node (“child” to the PortPlusNotificationDetailsExluding HazMatInfo_LIST element) |
| ATA | 1 | latest update of the attribute registered in SSN EIS notification database for the PortPlus notification identified by the ShipCallID |
| Anchorage | 0-1 |  |
| DepartureNotification Details | 0-1 | DepartureNotification Details element node. (“child” to the PortPlusNotificationDetails ExludingHazMatInfo_LIST element) |
| ATD | 1 | latest update of the attribute registered in SSN EIS notification database for the PortPlus notification identified by the ShipCallID |
| HazmatConfirmation | 1 | HazmatConfirmation second level-element node (“child” to the PortPlusNotificationDetailsExludingHazMatInfo_LIST elementt) |
| HazmatOnBoardYorN | 1 | latest update of the attribute registered in SSN EIS notification database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| PoBasinHazmatNotification | 0-1 | Mandatory in case the value of attribute HazmatOnBoardYorN is Y |
| HazmatInformation | 0-1 | HazmatInformation element node – child to QueryResults element |
| HazmatSummary | 0-1 | HazmatCargoInformation second level-element node (“child” to (“child” to the HazmatInformation element) |
| DGclassification | 0-∞ | latest update of the attribute registered in SSN EIS notification database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| INFShipClass | 0-1 |  |
| < TBD1 |  |  |
| < TBDx> |  |  |
| HazmatNotificationDetailsViaXML | 0-1 | HazmatNotificationDetailsViaXML element node(“child” to the HazmatInformation element) Mandatory if HazmatNotificationDetailsViaPhoneFaxOrURL not provided |
| CargoInformation | 1 | CargoInformation element node – Child to HazmatNotificationDetailsViaXML |
| PlacementOfGoods | 0-99 | PlacementOfGoods element describing the location of goods which are not in containers – child of CargoInformation |
| LocationOnBoardGoods | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| DPG | 1-9999 | DPG element node describing each type of dangerous goods on board - child of CargoInformation |
| TechnicalName | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| UNNumber | 1 |  |
| IMOHazardClass | 1 |  |
| WeightGross | 0-1 | WeightGross element specifying the gross weight of the dangerous good. Mandatory if WeightNet not present. – Child of DPG |
| UnitOfMeasurementGross | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| GrossQuantity | 1 |  |
| WeightNet | 0-1 | WeightNet element specifying the net weight of the dangerous good. Mandatory if WeightGross not present. - – Child of DPG |
| UnitOfMeasurementNet | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| NetQuantity | 1 |  |
| PlacementOfGoodsInContainer | 0-99 | PlacementOfGoodsInContainer element specifying the placement of the container which contains the dangerous good – child of CargoInformation |
| TransUnitId | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| LocationOnBoardContainer | 1 |  |
| DPG | 1-9999 | DPG element node describing each type of dangerous goods on board. – Child of PlacementOfGoodsInContainer |
| TechnicalName | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| UNNumber | 1 |  |
| IMOHazardClass | 1 |  |
| WeightGross | 0-1 | WeightGross element specifying the gross weight of the dangerous good. Mandatory if WeightNet not present. – Child of DPG |
| UnitOfMeasurementGross | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| GrossQuantity | 1 |  |
| WeightNet | 0-1 | WeightNet element specifying the net weight of the dangerous good. Mandatory if WeightGross not present. – Child of DPG |
| UnitOfMeasurementNet | 1 | latest update of the attribute registered in NCA database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| NetQuantity | 1 |  |
| HazmatNotificationDetailsViaPhoneFaxOrURL | 0-1 | HazmatNotificationDetailsViaPhoneFaxOrURL element node(“child” to the HazmatInformation element) Mandatory if HazmatNotificationDetailsViaXML not provided If specified, that means the Hazmat notification details are available as a document on a web server (UrlDetails must then be specified) or via a phone/fax (ContactDetails must then be specified). |
| UrlDetails | 0-1 | Second level element node (child to HazmatNotificationDetailsViaPhoneFaxOrURL indicating the type and the url of the document containing the notification details (if the data provider will store the document on a local web server). Not allowed if ContactDetails specified. |
| Url | 1 | latest update of the attribute registered in SSN EIS notification database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| DocType | 1 |  |
| ContactDetails | 0-1 | Second level element node (child to HazmatNotificationDetailsViaPhoneFaxOrURL indicating the contact details to obtain the notification details (if the data provider can only provide the information via phone or fax). Not allowed if UrlDetails specified. |
| LastName | 0-1 | latest update of the attribute registered in SSN EIS notification database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| FirstName | 0-1 |  |
| LoCode | 1 |  |
| Phone | 1 |  |
| Fax | 1 |  |
| EMail | 0-1 |  |
| CargoManifest | 1 | CargoManifest second level element (“child” to the (“child” to the HazmatInformation element)) The cargo manifest should be available as a document on a web server (UrlDetails must then be specified) or via a phone/fax (ContactDetails must then be specified). |
| UrlDetails | 0-1 | Third level element node (child to CargoManifest) element node indicating the type and the url of the document containing the cargo manifest (if the data provider will store the document on a local web server). Not allowed if ContactDetails specified. |
| Url | 1 | latest update of the attribute registered in SSN EIS notification database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| DocType | 1 |  |
| ContactDetails | 0-1 | Third level element node (child to CargoManifest) indicating the contact details to obtain the cargo manifest (if the data provider can only provide the information via phone or fax). Not allowed if UrlDetails specified. |
| LastName | 0-1 | latest update of the attribute registered in SSN EIS notification database for the voyage towards the PortOfCall identified in the voyage element of this response message |
| FirstName | 0-1 |  |
| LoCode | 1 |  |
| Phone | 1 |  |
| Fax | 1 |  |
| EMail | 0-1 |  |

| ##### Examples of request messages | The following examples illustrates the details of request messages of the SSN2MS_ShipCall_Res.xml type: |
| --- | --- |

Example in XML format to be included

In future versions of SSN – should the message structure will be used to accommodate additional queries to the SSN data, additional elements could be defined, e.g. an element identifying an  area of search

Additional attributes could be added in future versions of SSN

Additional elements could be added in future versions of SSN, e.g. in case it is decided that the same message structure will be used for requests for Incident reports, MRS data, etc

Additional attributes for future version of SSN to be used e.g. to restrict the query/ response to data concerning a subsidiary location within a port

Additional attributes could be added in future versions of SSN

Additional elements could be added in future versions of SSN, e.g. in case it is decided that the same message structure will be used for requests for Incident reports, MRS data, etc

In future versions of SSN – should the message structure will be used to accommodate additional queries to the SSN data, additional elements could be defined, e.g. an element identifying an  area of search

Additional attributes could be added in future versions of SSN

Additional elements could be added in future versions of SSN, e.g. in case it is decided that the same message structure will be used for requests for Incident reports, MRS data, etc

Additional attributes for future version of SSN to be used e.g. to restrict the query/ response to data concerning a subsidiary location within a port

Should it is decided that HazMat Summary information are to be kept at Member State level, the response should include such an element.

In future versions of SSN additional elements might be defined – should the message structure will be used for communicating the details of other type of notifications (e.g. MRS, Incident reports for identified vessels, etc

Additional attributes could be added in future versions of SSN

Additional elements could be added in future versions of SSN, e.g. in case it is decided that the same message structure will be used for requests for Incident reports, MRS data, etc

In future versions of SSN – should the message structure will be used to accommodate additional queries to the SSN data, additional elements could be defined, e.g. an element identifying an  area of search

Additional attributes could be added in future versions of SSN

Additional elements could be added in future versions of SSN, e.g. in case it is decided that the same message structure will be used for requests for Incident reports, MRS data, etc

Additional attributes for future version of SSN to be used e.g. to restrict the query/ response to data concerning a subsidiary location within a port

Should it is decided that HazMat Summary information are to be kept at Member State level, the response should include such an element.

In future versions of SSN additional elements might be defined – should the message structure will be used for communicating the details of other type of notifications (e.g. MRS, Incident reports for identified vessels, etc

Additional attributes could be added in future versions of SSN

Additional elements could be added in future versions of SSN, e.g. in case it is decided that the same message structure will be used for requests for Incident reports, MRS data, etc

Additional attributes for future version of SSN to be used e.g. to restrict the query/ response to data concerning a subsidiary location within a port

In future versions of SSN – should the message structure will be used to accommodate additional queries to the SSN data, additional elements could be defined, e.g. an element identifying an  area of search