SafeSeaNet WGO no 2	WG 2 / 7

Agenda item 8	 Rome, 23 April 2009

Lisbon, 20 April 2009

OPERATIONAL & TECHNICAL ISSUES

Feedback on the checking rules

Submitted by EMSA

| Executive summary | This document proposes two amendments on the checking rules agreed by the SSN Group during WS9 (21 & 22 may 2008). Proposals are based on feedback received from MS after tests |
| --- | --- |
| Action to be taken | According to section 4 |
| Related documents | - XML Reference Guide 1.65 - SSN 9.8.1 Data Quality Guidelines |

- Introduction

Based on the feedback received from Latvia, Belgium and the Netherlands and different studies carried out on the SSN databases it is proposed to review two checking rules before their effective implementation. The affected checking rules are:

- Sent At and ETA relation: Both Port and Hazmat notifications will be rejected in case the Sent At is subsequent to the ETA. The rule is Sent AT < ETA

- Ship’s name: Currently the system will only accept Arabic figures and Roman characters (alphabet).

- Sent At and ETA

From an operational point of view any notification, either Port or Hazmat, should arrive before the actual arrival to the ship at that port.

The proposal from the WGO regarding the procedure of “missing or mismatched information” is to request the MS that failed to notify on time to provide the information even if the vessel is already in port. The checking rule agreed would impede this late notification.

Apart from the above, recent studies have shown that a relative high number of notifications or their updates are sent “late”, this means they are sent after the ETA value. All this information is valuable, even if could be late for operational use, and would be lost if the rule is implemented.

Find below a table describing the statistics of MSS were the 1st SentAt > ETA (after the ship is in port) (data February 2009):

|  | 1st SentAt > ETA |  | SentAt > ETA |  | TOTAL |
| --- | --- | --- | --- | --- | --- |
| Country | Number of Rejected | % rejected | Number of Rejected | % rejected |  |
| Belgium | 289 | 4.0% | 3,107 | 42.6% | 7,290 |
| Cyprus | 31 | 15.0% | 33 | 15.9% | 207 |
| Denmark | 524 | 18.5% | 1,124 | 39.8% | 2,827 |
| Finland | 106 | 3.1% | 146 | 4.3% | 3,371 |
| France | 40 | 1.2% | 44 | 1.3% | 3,360 |
| Germany | 4 | 0.0% | 4 | 0.0% | 8,575 |
| Iceland | 3 | 6.3% | 3 | 6.3% | 48 |
| Ireland | 73 | 22.4% | 83 | 25.5% | 326 |
| Italy | 1,094 | 17.9% | 1,468 | 24.0% | 6,125 |
| Latvia | 10 | 1.5% | 10 | 1.5% | 686 |
| Lithuania | 240 | 16.6% | 448 | 31.0% | 1,443 |
| Malta | 0 | 0.0% | 0 | 0.0% | 127 |
| Netherlands | 908 | 9.5% | 1,185 | 12.4% | 9,591 |
| Norway | 1 | 0.0% | 1 | 0.0% | 5,096 |
| Poland | 0 | 0.0% | 0 | 0.0% | 1,733 |
| Portugal | 39 | 3.4% | 68 | 5.9% | 1,144 |
| Romania | 88 | 21.2% | 126 | 30.3% | 416 |
| Slovenia | 7 | 4.7% | 7 | 4.7% | 148 |
| Spain | 845 | 10.7% | 858 | 10.9% | 7,887 |
| Sweden | 729 | 6.4% | 2,625 | 23.0% | 11,392 |
| United Kingdom | 6,236 | 37.3% | 6,585 | 39.4% | 16,722 |
| TOTAL | 11,267 | 12.7% | 17,925 | 20.3% | 88,514 |

Notifications referring to the same Ship Call have been grouped by applying the rule:

If  “same IMO = same NextPortCall = same ETA” then “same Ship Call” .

The above table therefore does not consider the updates on the ETA.

This is only a rough calculation to assess the situation. For assessing properly this issue the ship call Id would be needed.

- Ship name

The DQ WG agreed on the following rule in the attribute “ship name”:

“Upon SOLAS, chapter I, part B, regulation 15 "Form Certificates", "the particulars inserted in the certificates shall be in Roman characters and Arabic figures". (FROM “A”  TO ”Z” AND FROM 0 TO 9)”.

Some MS have highlighted that some ship names contain special characters, such as dots or dashes. The MSS with the co-operation of the development sector of SSN has carried out a study on the special characters employed in the names, either on SSN or in other reference databases such as LMIU and LRF.

| Character | Number of ship names including the character (SSN, some 52000 ships) | Number of ship names including the character (combined list of SSN+LMIU+LRF, some 51500 ships) |
| --- | --- | --- |
| Dot (.) | 1476 | 3004 |
| Dash (-) | 1162 | 1560 |
| Backslash (\) | 26 | 3 |
| Slash (/) | 181 | 64 |
| Apostrophe (‘) | 76 | 96 |
| Inverted comas (“) | 16 | 3 |
| Underscore (_) | 56 | 7 |

Some ships’ names contain special characters such as dots and dashes that may be considered valid such as: “Volgo-Neft” or “Volgo-Balt”, “C’est la vie” or “D’amato”, “Hanna B.” or “Amin A.”.

Other characters should not be accepted:

- Slash: is mainly used in the acronym of the ship type such as M/V for motor vessel, R/V for rescue boat, S/V for sailing vessel, etc.

- Inverted comas: used mainly to quote the ship name (i.e., “superfast”).

- Brackets: usually employed to insert former names, etc.

- Proposal

The WGO is invited to agree on the following amendments to the checking rules for SSN V 1.9.1:

- SenAt > ETA: instead of rejecting the notification, a warning will be included in the “Status message” sent in the receipt message from EIS. The proposed wording is: “Warning: This notification contains a SentAt subsequent of the ETA”

- Ship name: the new wording in the XMLRG for all the attributes “shipname” would be: “Upon SOLAS, chapter I, part B, regulation 15 "Form Certificates", "the particulars inserted in the certificates shall be in Roman characters and Arabic figures". (FROM “A”  TO ”Z” AND FROM 0 TO 9). Also the following characters are accepted: dot (.), dash (-) and apostrophe (´) as they are employed in certain actual ship names.”