- Objective

To inform or to confirm, to MSs, that a single hull tankers has been identified in one of their notification.

To receive a feedback from MS about the compliance of this SHT regarding EC regulation 417/2002 amended.

- Background

The purpose of the EC regulation 417/2002 amended is to establish an accelerated phasing-in scheme for the application of the double hull or equivalent design requirements of the MARPOL 73/78 Convention to single hull oil tankers, and to ban the transport to or from ports of the Member States of heavy grades of oil in single-hull oil tankers.

SSN version 1,9,1,2 and STIRES should include a “ship activity tracking tool” allowing each MS to automatize and customize this early warning for any vessel or group of vessels. This procedure is considered as temporary.

- Procedure Overview

- Actions, description and responsible within the procedure

| Action | Description/ Remarks | Responsible (NCA, LCA, EMSA –MSS) | Reaction time |
| --- | --- | --- | --- |
| - NCA 24/7 receives a warning message in the SSN2MS_Receipt message, or - NCA 24/7 receives a warning from Maritime Support Services (MSS). | (Point 1 applicable if warnings issued by the SSN system are being 24/7 monitored) |  |  |
| - NCA 24/7 forwards the warning to the Local Competent Authority (Port, VTS, Coastal Station- CST) responsible for monitoring of the vessels posing potential risk. |  | NCA 24/7, LCA | ASAP, or as per local procedures |
| - NCA 24/7 forwards the request for the inquiry on the status of the vessel (scheduled phase-out and cargo on board) to the responsible Local Competent Authority (Port, VTS, Coastal Station- CST). | LCA responsible for the inquiry may be the same as the one responsible for the monitoring of the vessels posing a potential risk. | NCA 24/7, LCA | asap, or as per local procedures |
| - LCA obtains the information on the status of the vessel and cargo carried on board. |  | LCA | asap, or as per local procedures |
| - NCA receives information form LCAs and acknowledge EMSA on the result of the inquiry. | Expected information are, among others: - Status of the vessel (type of the vessel, IMO category and planned phase-out date if SHT) - Cargo carried on board - Activity of the vessel (intended port call, etc.) | NCA 24/7 | 24 hrs |
| - MSS receives feedback for the Member State. |  |  |  |
| - In cases when the tanker was converted to a different type (bulker, heavy load carrier, double hull), EMSA list of the SHT will be updated accordingly. |  | EMSA | Asap in SSN List updated once a month |

- Instructions and guidelines

Non applicable

- Templates

| Information provided to/for operators before hail 1. Ship name: NAME | 2. Call sign : XXXX | 3. IMO-ident: XXXXX |
| --- | --- | --- |
| 4. Nationality (Flag-state)/ Port of registry: XXXXXX |  |  |
| 5. Type and Class of vessel: XXXXXX |  |  |
| 6. Tonnage: GRT: XXXX DWT : XXXX |  |  |
| 7. Draught: AIS: xx,x m VTS/other: m |  |  |
| 8. Route: Departure: Destination: |  |  |
| 9. Hazmat information from SafeSeaNet: Yes ⁭ No ⁭ |  |  |
| 10. AIS POS: XX XX N/S ; XXX XX E/W at YYYY:MM:DD hh:mm:ss UTC |  |  |
| 9. Information regarding the vessels previous SHT history: |  |  |
| 11. Remarks before hailing: e.g. no pilot, the vessel is registered on the EMSA SHT list |  |  |
| 12. Hailing Position: |  |  |
| Hail Procedure and wording 13. This is the (authority), which has the responsibility for monitoring the (national) waters and to control that the vessels apply to international rules and regulations. This includes transportation of heavy grade oil in single hull tankers. In relation to these rules, we require some additional information. |  |  |
| 14. Is your Vessel a Single Hull Tanker: YES (Go to question 15) NO (Go to question 23) |  |  |
| 15. If information from single hull handler “can you confirm that your vessel is a IMO category 1,2 or 3 tanker” If no information from the single hull handler “what is your vessels IMO category” |  |  |
| 16. Is your vessel loaded or in ballast condition: loaded (Go to question 17) Ballast (Go to question 19) |  |  |
| 17. What is the type of the cargo: - IF OIL (Go to question 18) - IF BITUMEN, TAR and their emulsions = HGO (Go to question 20) - Anything else (Go to question 19) |  |  |
| 18. What is the density of the cargo at 15oC (higher than 900 kg/m3 = HGO): |  |  |
| 19. What is the Kinematic Viscosity at 50 oC (higher than 180 mm2/s = HGO):of your cargo |  |  |
| 20. What is your next port of call/destination?: |  |  |
| 21. Do you intend to load heavy grade oil at your next port of call?: |  |  |
| 22. Are you aware of the MARPOL Annex I regulations concerning the banning of carriage of heavy grade oil in single hull tankers? |  |  |
| 23. Thank you for your cooperation and information (end Call) |  |  |

- Tools and reference documents

| Required Tools/reference documents | How to access it? | Resp for the manual/tool. |
| --- | --- | --- |
| REGULATION (EC) No 417/2002 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL | http://eur-lex.europa.eu/Result.do?T1=V2&T2=2002&T3=417&RechType=RECH_consolidated&Submit=Search | / |