SafeSeaNet Workshop no. 11	SSN 11/3/11

Agenda item III	11-12 June 2009

Lisbon, 25 May 2009

German proposal following the 1st and 2nd WGT meeting.

Follow up of the SSN WGT - 2nd Meeting on 14 May 2009

Submitted by Germany

| Executive summary | This document introduces a proposal of an other possibility to send the requested notifications in order to comply with the Recast PSC Directive through SSN complying with the present XMLRefGuide 1.64 |
| --- | --- |
| Action to be taken | As per paragraph 4 |
| Related documents | Workshop report of the SSN WGT1 (The Hague, 18 February 2009) Workshop report of the SSN WGT2 (Lisbon, 14 May 2009) |

# INTRODUCTION

The recast Directive on Port State Control (PSC) requires Member States (MS) to provide information to THETIS (the future PSC information system) through SafeSeaNet (SSN). These requested data are:

- ATA (Actual Time of Arrival) of a vessel at the current port

- ATD (Actual Time of Departure) of a vessel at the current port

The information of the 24 hours PreArrival, already provided to SSN (directive 2002/59 EC), will also be forwarded to THETIS.

This paper will introduce a proposal for a more practical possibility to send the notifications requested by the Recast PSC Directive through SSN complying with the present XMLRefGuide 1.64

# Analyese of the proposed port plus Message

As discussed in several meetings especially in the data quality meetings there is a need of reliable data from MS. In the scope of this idea EMSA introduced in the 1. WGT in The Hague the ShipRefId as a mandatory attribute.

In the 1. WGT meeting in The Hague, there was a proposal by EMSA to build up XML notifications for each new element with the mandatory attribute ShipRefId. In the meeting was no common agreement regarding the need of a mandatory ShipRefID.

The new proposal in the 2nd WGT in Lisbon regarding the port plus message could only be managed by introducing the mandatory ShipRefId. The need to manage the new requirements of the recast PSC Directive, the port message and the Hazmat message in only one message (Port plus message with the obvious needed mandatory attributes) regarding the short time for legislation, budgeting, building up the business rules, IT-realisation, commissioning,… is not obvious.

For reliable data the ShipRefId has to be managed on national level. Each MS has to build up business rules and IT-structures to exchange this unique ShipRefId between all data provider (VTS, Ports, shipowners, agents, PSC officers,..) to be aware that for each specific ship call only this unique ShipRefId is used.

This will be a big challenge for the next years in most MS to establish the appropriate communication structures and business rules.

At the time now, there is a need to realise the needed notifications for THETIS in an more easy and regarding timeframe for realisation reachable way.

# Proposal of using serveral messages to be compiant with the Recast PSC Directive

As an agreed follow-up action of the 2st WGT, Germany will propose two notifications for the data requested by the Recast PSC Directive through SSN complying with the present XMLRefGuide 1.64.

#### MS2SSN_Arrival_Not.xml message

| ##### Introduction | The MS2SSN_Arrival_Not.xml message is sent by a Member State to SafeSeaNet in order to notify the actual arrival of a ship to a European port. |
| --- | --- |

| ##### Notification details | This notification already contains all its detailed information required to be passed to THETIS in accordance with the legal requirements of the PSC recast directive. |
| --- | --- |

| ##### Message description | ##### The following table describes the XML message used for the transaction. |
| --- | --- |

| Item | Occ | Type | Len | Description |
| --- | --- | --- | --- | --- |
| Header | 1 |  |  | Header Node |
| Version | 1 | Text | 3 | SafeSeaNet request current version (‘x.x’) |
| TestId | 0-1 | Text | 1-8 | Test Case identification. Only useful for testing. |
| MSRefId | 1 | Text | 1-36 | Reference identifier given by the original caller. It will be inserted back by SafeSeaNet in the MSRefId attribute of the SSN_Receipt.xml response. The MSRefId must be unique. |
| SentAt | 1 | DT | 19 | Notification creation date and time (ISO 8601 UTC format) All the time/date related attributes are in UTC. If local time is used MS application has to adjust the time to UTC. SentAt< ATA < ETD in UTC. |
| From | 1 | Text | 3-15 | The name of the originator of the message |
| To | 1 | Text | 3-15 | The name of the recipient of the message (‘SSN’). |
| Body | 1 |  |  | Body Node |
| Notification | 1 |  |  | Notification element node |
| VesselIdentification | 1 |  |  | VesselIdentification element node The vessel identification attributes (IMO number, MMSI, Call Sign, ship name) have to be checked against a reference ship database |
| IMONumber | 0-1 | Text | 7 | IMO number of the vessel. Mandatory if MMSINumber not given IMO number – IMO Res A.600 (15)- has to be checked if not existing in the reference DATABASE. |
| MMSINumber | 0-1 | Text | 9 | MMSI number of the vessel. Mandatory if IMONumber not given MID according to the ITU regulation. |
| CallSign | 0-1 | Text | 1-7 | Call sign of the vessel - Optional |
| ShipName | 0-1 | Text | 1-35 | Name of the vessel Upon SOLAS, chapter I, part B, regulation 15 "Form Certificates", "the particulars inserted in the certificates shall be in Roman characters and Arabic figures". (FROM “A” TO ”Z” AND FROM 0 TO 9) |
| VoyageInformation | 1 |  |  | VoyageInformation element node |
| PortOfCall | 1 | Text | 5 | This attribute indicates the actual port of call, identified by its LOCODE |
| ATA | 1 | DT | 19 | Date and time in ISO 8601 UTC format (YYYY-MM-DDThh:mm:ss) of the actual time of arrival at port of call. ATA <”Sent at” in UTC or ATA=NULL ATA<ETD in UTC |
| ETD | 0-1 | DT | 19 | Date and time in ISO 8601 UTC format (YYYY-MM-DDThh:mm:ss) of the estimated time of departure from the port of call (ETD > ATA). |
| PossibleAnchorage | 0-1 | ENUM | 1 | Enumeration indicating whether the call is at anchorage. Possible values: 1: Ship in anchorage at “sent-at” time 0: Ship is not in anchorage at “sent-at” time |
| PositionInPort | 0-1 | Text | 15 | The structure of the subsidiary LOCODE should follow the structure agreed by M.S. during the SSN7 workshop Element; Length. Fairway section number; 0-5. Terminal code; 0-5. Fairway hectometer; 0-5. |
| TotalPersonsOnBoard | 0-1 | Int |  | Total number of persons aboard. 99999 if actually unknown. Dots and commas are not allowed. |
| SubseqPortOfCall | 0-1 | Text | 5 | This attribute indicates the port of subsequent ship call identified by its LOCODE |

The following schema, which is in-line with the current SSN architecture, outlines the expected synchronous flow of XML messages related to pre-arrival notifications sent via  XML. A SSN_Receipt XML message will always be returned as response to a notification.

| Member State (data provider) |  | SafeSeaNet |  |
| --- | --- | --- | --- |
| MS2SSN_<Arrival>_Not |  |  |  |
| SSN_Receipt |  |  |  |

#### MS2SSN_Departure_Not.xml message

| ##### Introduction | The MS2SSN_Departure_Not.xml message is sent by a Member State to SafeSeaNet in order to notify the actual departure of a ship from a European port. |
| --- | --- |

| ##### Notification details | This notification already contains all its detailed information required to be passed to THETIS in accordance with the legal requirements of the PSC recast directive. |
| --- | --- |

| ##### Message description | ##### The following table describes the XML message used for the transaction. |
| --- | --- |

| Item | Occ | Type | Len | Description |
| --- | --- | --- | --- | --- |
| Header | 1 |  |  | Header Node |
| Version | 1 | Text | 3 | SafeSeaNet request current version (‘x.x’) |
| TestId | 0-1 | Text | 1-8 | Test Case identification. Only useful for testing. |
| MSRefId | 1 | Text | 1-36 | Reference identifier given by the original caller. It will be inserted back by SafeSeaNet in the MSRefId attribute of the SSN_Receipt.xml response. The MSRefId must be unique. |
| SentAt | 1 | DT | 19 | Notification creation date and time (ISO 8601 UTC format) All the time/date related attributes are in UTC. If local time is used MS application has to adjust the time to UTC. SentAt< ATD in UTC. |
| From | 1 | Text | 3-15 | The name of the originator of the message |
| To | 1 | Text | 3-15 | The name of the recipient of the message (‘SSN’). |
| Body | 1 |  |  | Body Node |
| Notification | 1 |  |  | Notification element node |
| VesselIdentification | 1 |  |  | VesselIdentification element node The vessel identification attributes (IMO number, MMSI, Call Sign, ship name) have to be checked against a reference ship database |
| IMONumber | 0-1 | Text | 7 | IMO number of the vessel. Mandatory if MMSINumber not given IMO number – IMO Res A.600 (15)- has to be checked if not existing in the reference DATABASE. |
| MMSINumber | 0-1 | Text | 9 | MMSI number of the vessel. Mandatory if IMONumber not given MID according to the ITU regulation. |
| CallSign | 0-1 | Text | 1-7 | Call sign of the vessel - Optional |
| ShipName | 0-1 | Text | 1-35 | Name of the vessel Upon SOLAS, chapter I, part B, regulation 15 "Form Certificates", "the particulars inserted in the certificates shall be in Roman characters and Arabic figures". (FROM “A” TO ”Z” AND FROM 0 TO 9) |
| VoyageInformation | 1 |  |  | VoyageInformation element node |
| PortOfCall | 1 | Text | 5 | This attribute indicates the actual port of call, identified by its LOCODE |
| ATD | 1 | DT | 19 | Date and time in ISO 8601 UTC format (YYYY-MM-DDThh:mm:ss) of the actual time of departure from the port of call |
| SubseqPortOfCall | 0-1 | Text | 5 | This attribute indicates the port of subsequent ship call. identified by its LOCODE |
| ETA_SubseqPortOfCall | 0-1 | Text | 19 | Date and time in ISO 8601 UTC format (YYYY-MM-DDThh:mm:ss) of ETA to the subsequent port of call This attribute indicates the ETA at the subsequent port of call |

The following schema, which is in-line with the current SSN architecture, outlines the expected synchronous flow of XML messages related to arrival notifications sent via  XML. A SSN_Receipt XML message will always be returned as response to a notification.

| Member State (data provider) |  | SafeSeaNet |  |
| --- | --- | --- | --- |
| MS2SSN_<Departure>_Not |  |  |  |
| SSN_Receipt |  |  |  |

# Action required

The SSN group is invited to consider and approve the above explained proposal and

to request EMSA to report on the changes to be undertaken in the SafeSeaNet system

resulting from the new proposal in order to clarify the impact of the changes to MS

applications.

The 72 hours PreArrival information is only defined in the Recast PSC Directive (article 6, Annex III) to be send to the MS (for ship eligible to an expanded inspection). Germany will clarify this item (the need to be send to SSN and then to THETIS) in the next meeting of MSCC42.